RPM sensing and calculation:

Timer 2 ticks every 1 microsecond and is configured to “Input Capture Mode” which records the timer value every time an encoder pulse arrives. DMA is also enabled so that these captured values can be automatically copied into a buffer without needed to occupy the CPU. Each time DMA stores a new timestamp, the HAL_TIM_IC_CaptureCallback() function is called, which finds the difference between the newest timestamp and the timestamp from the WINDOW definition (currently 6, with SLOTS_ON_DISK being 24) pulses ago. This value gives the time it took for ¼ revolution, which is saved to a variable and then a flag is set which notifies the while(1) loop to calculate the RPM. So the DMA interrupt only calculates time, the main while loop calculates the RPM.

In the while(1) loop, when the flag for a new calculation is set, the CPU does some math to calculate the RPM based on the time it took for a ¼ revolution. It then implements a median filter, which looks at the last 3 RPM values and takes the median of those values as the current RPM to reject outliers such as spikes from any issues with the sensor readings. It then also smooths the output slightly using alpha filtering which gives some weight to the past values instead of going 100% with the new values. It then saves a timestamp of the current time which is used in the timeout logic. If there hasn’t been an RPM calculation in a certain amount of time, that means there are no new pulses and disk has stopped spinning, so the RPM is set to 0.